AN-Najah National University

Factuality of Engineering

## Electrical Engineering Department

GRADUATION PROJECT:

Digital Steam Iron

Prepared by :

Allam Zatar

Alaa Rawajby

Presented to :

-

A.omar al tamimi

2010-05-11

Digital Steam Iron
The project relates to a digital steam iron comprising a digital heat generator. This unit will be based on microcontroller system to accurately adjust the temperature according to the user requirements. 
It is well known that the conventional steam iron has many disadvantages in terms of temperature instability, which often increase the risk of damaging ironed clothes. Clothes can also be damaged if the iron is left laid flat on them for a period of time. 
The digital steam iron will propose two solutions for the above mentioned problems. The first solution is to introduce an accurate temperature control unit based on microcontroller system. This unit provides a range of temperature levels selected manually using either pushbuttons or barcode reader. It should also maintain the level of temperature throughout the ironing process.
The second solution is to introduce an automatic cut off unit based on vibration sensor element. This unit will be utilized to determine whether the steam iron is being used. As mentioned earlier. This feature prevents ironed clothes from accidental damage which occur as a result of having the iron left laid flat on them.

Project  Objective :

to accurately adjust the temperature according to the user requirements. *

*introduce an automatic cut off unit based on accelerometer sensor element

Conventional steam iron

An electric steam iron includes a water tank, a bottom plate, a heating device which is used to heat the bottom plate, a steam generator having a boiling chamber, a heating device to heat the boiling chamber and a temperature controller, with the inlet of the boiling chamber being in fluid communication with the water tank and the outlet of the boiling chamber being in fluid communication with the outlet opening of the bottom plate. The heating devices of the bottom plate and the steam generator are connected in series with their respective heating circuit, so that there is sole control of the steam generator, such that when using the iron again, it can spray steam directly without warming up the steam generator, thereby eliminating the warming up time of the steam generator and greatly speeding up boiling of the water.

In our project there is a microcontroller embedded in the steam iron to ensure that a constant temperature will be produced when ironing any material. Users can set the appropriate temperature for ironing any material. Before ironing, simply select from a list of materials on the lcd of the steam iron. Automatic switch off when we left it over clothes for some time due to vibration sensor

Microcontroller:

Arduino is a tool for making computers that can sense and control more of the physical world than your desktop computer. It's an open-source physical computing platform based on a simple microcontroller board, and a development environment for writing software for the board.

Arduino can be used to develop interactive objects, taking inputs from a variety of switches or sensors, and controlling a variety of lights, motors, and other physical outputs. Arduino projects can be stand-alone, or they can be communicate with software running on your computer (e.g. Flash, Processing, MaxMSP.) The boards can be assembled by hand or purchased preassembled; the open-source IDE can be downloaded for free.

The Arduino programming language is an implementation of Wiring, a similar physical computing platform, which is based on the Processing multimedia programming environment.

Why Arduino?

There are many other microcontrollers and microcontroller platforms available for physical computing. Parallax Basic Stamp, Netmedia's BX-24, Phidgets, MIT's Handyboard, and many others offer similar functionality. All of these tools take the messy details of microcontroller programming and wrap it up in an easy-to-use package. Arduino also simplifies the process of working with microcontrollers, but it offers some advantage for teachers, students, and interested amateurs over other systems:

- Cross-platform - The Arduino software runs on Windows, Macintosh OSX, and Linux operating systems. Most microcontroller systems are limited to Windows.

- Simple, clear programming environment - The Arduino programming environment is easy-to-use for beginners, yet flexible enough for advanced users to take advantage of as well. For teachers, it's conveniently based on the Processing programming environment, so students learning to program in that environment will be familiar with the look and feel of Arduino

- Open source and extensible software- The Arduino software and is published as open source tools, available for extension by experienced programmers. The language can be expanded through C++ libraries, and people wanting to understand the technical details can make the leap from Arduino to the AVR C programming language on which it's based. SImilarly, you can add AVR-C code directly into your Arduino programs if you want to.

- Open source and extensible hardware - The Arduino is based on Atmel's ATMEGA8 and ATMEGA168 microcontrollers. The plans for the modules are published under a Creative Commons license, so experienced circuit designers can make their own version of the module, extending it and improving it. Even relatively inexperienced users can build the breadboard version of the module in order to understand how it works and save money.

Accelerometer

An accelerometer is a device that measures proper acceleration, the acceleration experienced relative to freefall.

Single- and multi-axis models are available to detect magnitude and direction of the acceleration as a vector quantity, and can be used to sense position, vibration and shock. Micromachined accelerometers are increasingly present in portable electronic devices and video game controllers, to detect the position of the device or provide for game input.

## Physical principles

An accelerometer measures proper acceleration, which is the acceleration it experiences relative to freefall, and is the acceleration that is felt by people and objects. Such accelerations are popularly measured in terms of g-force. Put another way, at any point in spacetime the equivalence principle guarantees the existence of a local inertial frame, and an accelerometer measures the acceleration relative to that frame.[1]

As a consequence an accelerometer at rest relative to the Earth's surface will indicate approximately 1 g upwards, because any point on the earth's surface is accelerating upwards relative to the local inertial frame, which would be the frame of a freely falling object at the surface. To obtain the pure acceleration due to motion with respect to the Earth, this "gravity offset" must be subtracted. This is generally true of any gravitational field, since gravity does not produce proper acceleration, and an accelerometer is not sensitive to it, and cannot measure it directly.

The reason for the appearance of a gravitational offset is Einstein's equivalence principle[2], which states that the effects of gravity on an object are indistinguishable from acceleration of the reference frame. When held fixed in a gravitational field by, for example, applying a ground reaction force or an equivalent upward thrust, the reference frame for an accelerometer (its own casing) accelerates upwards with respect to a free-falling reference frame. The effect of this reference frame acceleration is indistinguishable from any other acceleration experienced by the instrument, so that an accelerometer cannot tell the difference between sitting in a rocket on the launch pad, and being in the same rocket accelerated at 1 g by the engines, in deep space.

For similar reasons, an accelerometer will read zero during any type of free fall. This includes use in a coasting spaceship in deep space far from any mass, a spaceship orbiting the Earth, an airplane in a parabolic "zero-g" arc, or any free-fall in vacuum. An example would be a free-fall from such a high altitude that atmospheric effects could initially be neglected.

However such a situation does not include a (non-free) fall, in which air resistance produces drag forces that reduce the acceleration, until constant terminal velocity is reached, at which point the device would once again indicate 1 g acceleration upwards, produced by the drag. The physical counterpart to this effect is that a skydiver, upon reaching such a terminal velocity, does NOT feel as though he or she were in "free-fall", but rather experiences a feeling similar to being supported (at 1 g) on a "bed" of uprushing air.

Acceleration is quantified in the SI unit metres per second per second (m/s2), in the cgs unit gal (Gal), or popularly in terms of g-force (g).

For the practical purpose of finding the acceleration of objects with respect to the Earth, such as for use in an inertial navigation system, a knowledge of local gravity is required. This can be obtained either by calibrating the device at rest[3], or from a known model of gravity at the approximate current position.

MAX6675

The MAX6675 performs cold-junction compensation and digitizes the signal from a type-K thermocouple. The data is output in a 12-bit resolution, SPI-compatible, read-only format.

This converter resolves temperatures to 0.25°C, allows readings as high as +1024°C, and exhibits thermocouple accuracy of 8 LSBs for temperatures ranging from 0°C to +700°C

Thermocouples - An Introduction
what is a thermocouple sensor?
A thermocouple is a sensor for measuring temperature. It consists of two dissimilar metals, joined together at one end. When the junction of the two metals is heated or cooled a voltage is produced that can be correlated back to the temperature. The thermocouple alloys are commonly available as wire. 

What are the different thermocouple types?
A thermocouple is available in different combinations of metals or calibrations. The four most common calibrations are J, K, T and E. There are high temperature calibrations R, S, C and GB. Each calibration has a different temperature range and environment, although the maximum temperature varies with the diameter of the wire used in the thermocouple. Although the thermocouple calibration dictates the temperature range, the maximum range is also limited by the diameter of the thermocouple wire. That is, a very thin thermocouple may not reach the full temperature range. to view a complete reference table for each thermocouple. The table includes international color codes for thermocouple alloys, temperature range and limits of error for almost every kind of thermocouple.

How do I choose a thermocouple type?
Because a thermocouple measures in wide temperature ranges and can be relatively rugged, thermocouples are very often used in industry. The following criteria are used in selecting a thermocouple:

- Temperature range

- Chemical resistance of the thermocouple or sheath material

- Abrasion and vibration resistance

- Installation requirements (may need to be compatible with existing equipment; existing holes may determine probe diameter)

How do I know which junction type to choose?
Sheathed thermocouple probes are available with one of three junction types: grounded, ungrounded or exposed (see graphic below:"Thermocouple Tip Styles"). At the tip of a grounded junction probe, the thermocouple wires are physically attached to the inside of the probe wall. This results in good heat transfer from the outside, through the probe wall to the thermocouple junction. In an ungrounded probe, the thermocouple junction is detached from the probe wall. Response time is slower than the grounded style, but the ungrounded offers electrical isolation (see table below).

The thermocouple in the exposed junction style protrudes out of the tip of the sheath and is exposed to the surrounding environment. This type offers the best response time, but is limited in use to dry, noncorrosive and nonpressurized applications.

What is response time?
A time constant has been defined as the time required by a sensor to reach 63.2% of a step change in temperature under a specified set of conditions. Five time constants are required for the sensor to approach 100% of the step change value. An exposed junction thermocouple is the fastest responding. Also, the smaller the probe sheath diameter, the faster the response, but the maximum temperature may be lower. Be aware, however, that sometimes the probe sheath cannot withstand the full temperature range of the thermocouple type.

Thermocouple Types

Beaded Wire Thermocouple
A beaded wire thermocouple is the simplest form of thermocouple. It consists of two pieces of thermocouple wire joined together with a welded bead. Because the bead of the thermocouple is exposed, there are several application limitations. The beaded wire thermocouple should not be used with liquids that could corrode or oxidize the thermocouple alloy. Metal surfaces can also be problematic. Often metal surfaces, especially pipes are used to ground electrical systems The indirect connection to an electrical system could impact the thermocouple measurement. In general, beaded wire thermocouples are a good choice for the measurement of gas temperature. Since they can be made very small, they also provide very fast response time.

Thermocouple Probe
A thermocouple probe consists of thermocouple wire housed inside a metallic tube. The wall of the tube is referred to as the sheath of the probe. Common sheath materials include stainless steel and Inconel. Inconel supports higher temperature ranges than stainless steel, however, stainless steel is often preferred because of its broad chemical compatibility. For very high temperatures, other exotic sheath materials are also available. View our line of high temperature exotic thermocouple probes.

The tip of the thermocouple probe is available in three different styles. Grounded, ungrounded and exposed. With a grounded tip the thermocouple is in contact with the sheath wall. A grounded junction provides a fast response time but it is most susceptible to electrical ground loops. In ungrounded junctions, the thermocouple is separated from the sheath wall by a layer of insulation. The tip of the thermocouple protrudes outside the sheath wall with an exposed junction. Exposed junction thermocouples are best suited for air measurement.

Surface Probe
Measuring the temperature of a solid surface is difficult for most types of temperature sensors. In order to assure an accurate measurement, the entire measurement area of the sensor must be in contact with the surface. This is difficult when working with a rigid sensor and a rigid surface. Since thermocouples are made of pliable metals, the junction can be formed flat and thin to provide maximum contact with a rigid solid surface. These thermocouples are an excellent choice for surface measurement. The thermocouple can even be built in a mechanism which rotates, making it suitable for measuring the temperature of a moving surface. Type K is ChromegaAlomega

Ac Phase controllers

Phase control , also called phase cutting, is a method of pulse width modulation (PWM) for power limiting, applied to AC voltages. It works by modulating a thyristor, SCR, triac, thyratron, or other such gated diode-like devices into and out of conduction at a predetermined phase of the applied waveform.

## Overview

Phase fired control is often used to control the amount of voltage, current or power that a power supply feeds to its load. It does this in much the same way that a pulse width modulated (PWM) supply would pulse on and off to create an average value at its output. If the supply has a DC output, its time base is of no importance in deciding when to pulse the supply on or off, as the value that will be pulsed on and off is continuous.

PFC differs from PWM in that it addresses supplies that output a modulated waveform, such as the sinusoidal AC waveform that the national grid outputs. Here, it becomes important for the supply to pulse on and off at the correct position in the modulation cycle for a known value to be achieved; for example, the controller could turn on at the peak of a waveform or at its base if the cycle's time base were not taken into consideration.

Phase fired controllers take their name from that fact that they trigger a pulse of output at a certain phase of the input's modulation cycle. In essence, a PFC is a PWM controller that can synchronise itself with the modulation present at the input.

Most phase fired controllers use thyristors or other solid state switching devices as their control elements. Thyristor based controllers may utilise Gate Turn Off (GTO) thyristors, allowing the controller to not only decide when to pulse the output on but also when to turn it off, rather than having to wait for the waveform to pass within the element's Zero Cross Point.

### Output reduction by bucking

A phase fired controller, like a buck topology switched-mode power supply, is only able to deliver an output maximum equal to that which is present at its input, minus any losses occurring in the control elements themselves. Provided the modulation during each cycle is predictable or repetitive, as it is on the national grid's AC mains, to obtain an output lower than its input, a phase fired control simply switches off for a given phase angle of the input's modulation cycle. By triggering the device into conduction at a phase angle greater than 0 degrees, a point after the modulation cycle starts, a fraction of the total energy within each cycle is present at the output.

### Applications

Previously, extremely expensive and heavy multi-tapped transformers were used as the supplies for such elements, with the corresponding winding tap being connected to the element to produce the desired temperature. This limited the temperature resolution to the number of tap combinations available. They often find their way into controllers designed for equipment such as electric ovens and furnaces.

In modern, usually high power, equipment, the transformer is replaced with phase fired controllers connecting the load directly to the mains, resulting in a substantially cheaper and lighter system. However, the method is usually limited to use in equipment that would be unrealistic without it. This is because removal of the mains transformer means that the load is in direct galvanic contact with the input. For industrial ovens and furnaces the input is often the national grid AC, which is itself galvanically referenced to the Earth. With the controller's output referenced to the Earth, a user need only be in contact with the Earth and one of the output terminals to risk receiving an electrical shock. With many high power pieces of equipment running from three phase 415 V, high current capable inputs and having the entirety of any metallic housing or framework present Earthed (grounded), this is a serious risk that must be assessed with care.

## . Power supply types

Power supplies for electronic devices can be broadly divided into linear and switching power supplies. The linear supply is a relatively simple design that becomes increasingly bulky and heavy for high current devices; voltage regulation in a linear supply can result in low efficiency. A switched-mode supply of the same rating as a linear supply will be smaller, is usually more efficient, but will be more complex.

### Linear power supply:

An AC powered linear power supply usually uses a transformer to convert the voltage from the wall outlet (mains) to a different, usually a lower voltage. If it is used to produce DC, a rectifier is used. A capacitor is used to smooth the pulsating current from the rectifier. Some small periodic deviations from smooth direct current will remain, which is known as ripple. These pulsations occur at a frequency related to the AC power frequency (for example, a multiple of 50 or 60 Hz).

The voltage produced by an unregulated power supply will vary depending on the load and on variations in the AC supply voltage. For critical electronics applications a linear regulator will be used to stabilize and adjust the voltage. This regulator will also greatly reduce the ripple and noise in the output direct current. Linear regulators often provide current limiting, protecting the power supply and attached circuit from overcurrent.

### Switched-mode power supply:

A switched-mode power supply (SMPS) works on a different principle. AC mains input is directly rectified without the use of a transformer, to obtain a DC voltage. This voltage is then sliced into small pieces by a high-speed electronic switch. The size of these slices grows larger as power output requirements increase.

The input power slicing occurs at a very high speed (typically 10 kHz — 1 MHz). High frequency and high voltages in this first stage permit much smaller step down transformers than are in a linear power supply. After the transformer secondary, the AC is again rectified to DC. To keep output voltage constant, the power supply needs a sophisticated feedback controller to monitor current draw by the load.

Modern switched-mode power supplies often include additional safety features such as the crowbar circuit to help protect the device and the user from harm. In the event that an abnormal high current power draw is detected, the switched-mode supply can assume this is a direct short and will shut itself down before damage is done. For decades PC power supplies have also provided a power good signal to the motherboard which prevents operation when abnormal supply voltages are present.

.

## Advantages and Disadvantages

The main advantage of this method is greater efficiency because the switching transistor dissipates little power when it is outside of its active region (i.e., when the transistor acts like a switch and either has a negligible voltage drop across it or a negligible current through it). Other advantages include smaller size and lighter weight (from the elimination of low frequency transformers which have a high weight) and lower heat generation due to higher efficiency.

Disadvantages include greater complexity, the generation of high-amplitude, high-frequency energy that the low-pass filter must block to avoid electromagnetic interference (EMI), and a ripple voltage at the switching frequency and the harmonic frequencies thereof.

## Digital control

By controlling analog circuits digitally, system costs and power consumption can be drastically reduced. What's more, many microcontrollers and DSPs already include on-chip PWM controllers, making implementation easy.

In a nutshell, PWM is a way of digitally encoding analog signal levels. Through the use of high-resolution counters, the duty cycle of a square wave is modulated to encode a specific analog signal level. The PWM signal is still digital because, at any given instant of time, the full DC supply is either fully on or fully off. The voltage or current source is supplied to the analog load by means of a repeating series of on and off pulses. The on-time is the time during which the DC supply is applied to the load, and the off-time is the period during which that supply is switched off. Given a sufficient bandwidth, any analog value can be encoded with PWM.

Figure 1 shows three different PWM signals. Figure 1a shows a PWM output at a 10% duty cycle. That is, the signal is on for 10% of the period and off the other 90%. Figures 1b and 1c show PWM outputs at 50% and 90% duty cycles, respectively. These three PWM outputs encode three different analog signal values, at 10%, 50%, and 90% of the full strength. If, for example, the supply is 9V and the duty cycle is 10%, a 0.9V analog signal results.

Figure 1. PWM signals of varying duty cycles

Figure 2 shows a simple circuit that could be driven using PWM. In the figure, a 9 V battery powers an incandescent lightbulb. If we closed the switch connecting the battery and lamp for 50 ms, the bulb would receive 9 V during that interval. If we then opened the switch for the next 50 ms, the bulb would receive 0 V. If we repeat this cycle 10 times a second, the bulb will be lit as though it were connected to a 4.5 V battery (50% of 9 V). We say that the duty cycle is 50% and the modulating frequency is 10 Hz.

Most loads, inductive and capacitative alike, require a much higher modulating frequency than 10 Hz. Imagine that our lamp was switched on for five seconds, then off for five seconds, then on again. The duty cycle would still be 50%, but the bulb would appear brightly lit for the first five seconds and off for the next. In order for the bulb to see a voltage of 4.5 volts, the cycle period must be short relative to the load's response time to a change in the switch state. To achieve the desired effect of a dimmer (but always lit) lamp, it is necessary to increase the modulating frequency. The same is true in other applications of PWM. Common modulating frequencies range from 1 kHz to 200 kHz.

### PWM controllers

Many microcontrollers include on-chip PWM controllers. For example, Microchip's PIC16C67 includes two, each of which has a selectable on-time and period. The duty cycle is the ratio of the on-time to the period; the modulating frequency is the inverse of the period. To start PWM operation, the data sheet suggests the software should:

- Set the period in the on-chip timer/counter that provides the modulating square wave

- Set the on-time in the PWM control register

- Set the direction of the PWM output, which is one of the general-purpose I/O pins

- Start the timer

- Enable the PWM controller

operate at clock rate frequencies as low as 4 kHz, as this is adequate for many typical applications, enabling low power consumption (milliwatts or microwatts). They will generally have the ability to retain functionality while waiting for an event such as a button press or other interrupt; power consumption while sleeping (CPU clock and most peripherals off) may be just nanowatts, making many of them well suited for long lasting battery applications. Other microcontrollers may serve performance-critical roles, where they may need to act more like a digital signal processor (DSP), with higher clock speeds and power consumption.

Microcontrollers are used in automatically controlled products and devices, such as automobile engine control systems, remote controls, office machines, appliances, power tools, and toys. By reducing the size and cost compared to a design that uses a separate microprocessor, memory, and input/output devices, microcontrollers make it economical to digitally control even more devices and processes. Mixed signal microcontrollers are common, integrating analog components needed to control non-digital electronic systems.

Function of MC:

Controlling of steam iron by Cutt off the steam iron when left it without moving by taking asignal from accelerometer  sensor

Controlling the temperature of the heater by received signal from temperature sensor

Controlling the LCD screen as simple fig. shown:

Conclusion:

This project solves the problem of conventional steam iron such as accurate temperature. And turn off ite Iron when miss it over the clothes for some time

This project gives us a lot of information to know we deal with analog and digital sensor and how we can use ac phase control.