- بسم الله الرحمن الرحيم

- An_Najah National University

- The Faculty of Engineering

- Computer Dept Eng

- GRADUATED PROJECT

PEN   PLOTTER

- Prepared by :ASMA Al SAQA

Pen plotter is a device that plot any bitmap  sent to by its computer  application by drawing a line by line data  .

Tools :

1-pic 16f877 as controller with its basic circuit .

2-ULN2003 and relays.

3-three DC motors :  to control the X and Y direction moving

4- DC motor to control the pen up and down moving .

MECHANICAL PART :

This part which take most of time and I make many design for it ,each time I have a problem I think  to solve it and reach to other design I try it ,I tried with at least five designs ,

Last I reach to ironic frame with two direction .

For the two direction first I use stepper motors  and after I find them from useless printers , and  program I find that the motors is very weak and  move the direction axis from one side , and so I think to use 2 steppers for each direction which put me in more problems , because the moving is not balance  ,because the steppers is too weak and also the torque  ; and because I’m not a mechanical engineer , I don’t know what to do to decrease the load  so ,and this is the most difficult decision because it need to redesign ,reprogramming  and change the hardware , it is to change the motors type from stepper to DC , to reach to the last design

HARDWARE PART

And the following to use three pens instead of one

SOFTWARE

I use PIC_C program and TR to program PIC16f877 .

I used Microsoft visual studio 2005 to design my application which treat with the following :

1-files "select bitmap":

a-I use files open browser to select a bitmap and return information about it as width and height and file name  .

op.ShowDialog();

s = op.FileName;

if (s == "") { }

else{

Bitmap b = new Bitmap(s);

w = b.Width;

h = b.Height;

int d = w * h;

txt.Text = s;

2-images:

a-after select a bitmap  I show it in picture box .

p1.Image = Bitmap.FromFile(s);

b-I show the bitmap name ,width and height in pixels .

te1.Text = w.ToString();

te2.Text = h.ToString();

c-I check the color of each pixel  and keep the values in an array ,if the color  is black I keep 1 in the array else I keep 0.

for (int i = 0; i < w; i++)

{

ar[i] = new byte[h];

//  ar[i][0]=2;

for (int j =0; j< h; j++)

{

//  ar[i][j] = b.GetPixel(i, j).B;

String blue = b.GetPixel(i, j).B.ToString();

String red = b.GetPixel(i, j).R.ToString();

String green = b.GetPixel(i, j).G.ToString();

if (blue == "0" && green=="0" && red=="0")

{

ar[i][j] = 1;

}

else

{

ar[i][j] = 0;

}

}

d-also I use it to increase size by 2 or 3 by using flags  .

to increase size by 2 :

if(s==""){

MessageBox.Show(" select a bitmap");

}

else{

size_flag = 1;

int w2=(w * 2);

te1.Text =w2.ToString();

w2=h*2;

te2.Text=w2.ToString();

}

}

to increase it by 3:

if (s == "")

{

MessageBox.Show(" select a bitmap");

}

else

{

size_flag =2;

int w2 = (w *3);

te1.Text = w2.ToString();

w2 = h * 3;

te2.Text = w2.ToString();

}

}

to  keep the original size :

{

if (s == "")

{

MessageBox.Show(" select a bitmap");

}

else

{

size_flag = 0;

te1.Text = w.ToString();

te2.Text = h.ToString();

}

}

- 3-serial port

a-when open port is pressed :I check if it is already open else I open it .

if (sp.IsOpen)

{

MessageBox.Show("the port is open");

}

else

{

sp.Open();

sp.BaudRate = 9600;

}

b-send data: I check if  the port  is open  and an image  is select then I send the data via serial port .

I check if the user want to draw the bitmap in its original size or to increase by two or three ,

If he want it as it I start to send data to the PIC which move motor s to  the position it must go and check if it is one ,draw it by move the pen down ,else will not draw .

If he want to increase size by two , I will draw  each pixel as tow pixel in X-direction and two pixel in Y-direction .

If he want to increase size by three ,I will draw each pixel as nine pixels ,three rows by three columns .

if (!sp.IsOpen)

{

MessageBox.Show("the port is not opened");

}

else

{

if (s == "")

{

MessageBox.Show("the bitmap is not opened");

}

else

{

byte[] a = new byte[1]; ;

Bitmap b = new Bitmap(s);

int w = b.Width;

int h = b.Height;

int flag = 0;

if(size_flag==0){

for (int i = 0; i < w; i++)

{

byte[] dd = new byte[1];

dd[0] = 2;

sp.Write(dd, 0, 1);

ta.Text= ta.Text+sp.ReadByte().ToString();

if (flag == 0)

{

flag = 1;

for (int j = 0; j < h; j++)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

else

{

flag = 0;

for (int j = h - 1; j >= 0; j--)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

}

}

else if (size_flag ==1)

{

for (int i= 0; i < w; i++)

{

byte[] dd = new byte[1];

dd[0] = 2;

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j = 0; j < h; j++)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j = h - 1; j >= 0; j--)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

}

else if (size_flag ==2)

{

for (int i = 0; i < w; i++)

{

byte[] dd = new byte[1];

dd[0] = 2;

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

if (flag == 0)

{

flag = 1;

for (int j = 0; j < h; j++)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j =h-1; j>=0; j--)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

;

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j = 0; j < h; j++)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

else

{

flag = 0;

for (int j = h - 1; j >= 0; j--)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j = 0; j<h; j++)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

sp.Write(dd, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

for (int j = h - 1; j >= 0; j--)

{

byte[] d = new byte[1];

d[0] = ar[i][j];

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

sp.Write(d, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

}

}

byte[] d1= new byte[1];

d1[0] = 3;

sp.Write(d1, 0, 1);

ta.Text = ta.Text + sp.ReadByte().ToString();

}

}

}

c-when close is pressed the application will end :

PIC C and PIC 16f877

In PICC code , I get the pixels sent to by serial port and check if it is zero or one then I will move and draw as following :

#include "dc-plot.h"

void pen_up_down(){

//int g=0;

//int o=0;

//int k=0;

restart_wdt();

output_low(PIN_D0);

//delay_ms(100);

//

output_high(PIN_D1);

restart_wdt();

delay_ms(15);

output_low(PIN_D1);

restart_wdt();

}

// void pen_up_up(){

// }

void pen_down_up(){

// int g=0;

// int o=0;

//int k=0;

output_low(PIN_D1);

output_high(PIN_D0);

restart_wdt();

delay_ms(15);

output_low(PIN_D0);

restart_wdt();

}

//  void pen_down_down(){

//  }

void step_l_to_r(){

int c=0;

int i=0;

output_low(PIN_D2);

output_high(PIN_D3);

restart_wdt();

delay_ms(50);

output_low(PIN_D3);

restart_wdt();

}

void step_r_to_l(){

int c=0;

int i=0;

output_low(PIN_D3);

output_high(PIN_D2);

restart_wdt();

delay_ms(50);

output_low(PIN_D2);

restart_wdt();

}

void step_up(){

output_low(PIN_c4);

output_high(PIN_c5);

restart_wdt();

delay_ms(50);

output_low(PIN_c5);

restart_wdt();

}

void step_down(){

output_low(PIN_c5);

output_high(PIN_c4);

restart_wdt();

delay_ms(50);

output_low(PIN_c4);

restart_wdt();

}

void main()

{

int c=0;

int x=0;

int y=0;

int i=0;

int j=0;

int step_dir;

int pen_flag;

char ch='4';

step_dir=0;   //0 l_to_R         1 r_to_l

pen_flag=0;//0 up 1 down

setup_adc_ports(NO_ANALOGS);

setup_adc(ADC_OFF);

setup_psp(PSP_DISABLED);

setup_spi(FALSE);

setup_counters(RTCC_INTERNAL,RTCC_DIV_1);

setup_timer_1(T1_DISABLED);

setup_timer_2(T2_DISABLED,0,1);

while(1){

ch=getc();

putc(ch);

while(ch!=3){

if(ch==0){

x++;   //to return to the original place

if(pen_flag==0){

//pen is up and must still up

if(step_dir==0){

step_l_to_r();}

else if(step_dir==1){

step_r_to_l();

}

}

else if(pen_flag==1){   //pen is down and must go up

if(step_dir==0){

step_l_to_r();}

else if(step_dir==1){

step_r_to_l();

}

pen_down_up();

pen_flag=0;

restart_wdt();

}}

else if(ch==1){

x++;

if(pen_flag==0){

if(step_dir==0){

restart_wdt();

step_l_to_r();}

else if(step_dir==1){

step_r_to_l();

restart_wdt();

}

restart_wdt();

pen_up_down();

//pen is up and must go down

restart_wdt();

pen_flag=1;

restart_wdt();

}

else if(pen_flag==1){    //pen is down and must still down

if(step_dir==0){

step_l_to_r();}

else if(step_dir==1){

step_r_to_l();

}

}}

else if(ch==2){

x=0;

step_down();

y++;

if(step_dir==0){

step_dir=1;}

else if(step_dir==1){

step_dir=0;

}

}

ch=getc();

putc(ch);

}

///////////////// to return to the original place ;;;;;;;;;;

}

}